AGREEMENT

between the Administrations of

Austria, the Czech Republic, Germany, Hungary, [Italy,] Liechtenstein, the Slovak Republic, Slovenia and Switzerland

concerning the allotment of preferential frequency blocks in the bands

450.000 – 457.400 MHz and 458.400 – 460.000 MHz

as well as

460.000 – 467.400 MHz and 468.400 – 470.000 MHz

Vienna, 3 December 2004

1.	Introduction

In the framework of the “Agreement (Berlin 2003)” the Administrations of Austria, the Czech Republic, Germany, Hungary, [Italy,] Liechtenstein, the Slovak Republic, Slovenia and Switzerland concluded this Agreement concerning the allotment of preferential frequencies in the bands 450.000 – 457.400 MHz and 458.400 – 460.000 MHz as well as 460.000 – 467.400 MHz and 468.400 – 470.000 MHz. The relevant provisions of  the “Agreement (Berlin 2003)” shall be applied unless otherwise laid down in this Agreement.

2. 	Principles – Background

2.1	The Administrations mentioned above deemed it necessary to conclude an agreement on the allotment of the preferential frequencies for narrow-band systems and the co-ordination principles for wide-band systems in the frequency bands 450.000 – 457.400 MHz and 458.400 – 460.000 MHz as well as 460.000 – 467.400 MHz and 468.400 – 470.000 MHz.

2.2	The co-ordination procedures shall be based on the concept of preferential frequencies (see Article 4.2 of the current version of the Agreement Berlin 2003).

2.3	The frequency bands 450.000 – 457.400 MHz and 458.400 – 460.000 MHz as well as 460.000 – 467.400 MHz and 468.400 – 470.000 MHz are split into preferential frequency blocks which shall be assigned equally between countries involved.

2.4		Operators shall have the possibility of using these frequencies in a different way in order to minimise interference and to achieve the most efficient use of the available spectrum. The provisions laid down in the relevant “Agreement between administrations concerned  regarding the approval of arrangements between operators” shall apply.

3.   Technical provisions

3.1	The division into preferential frequency blocks is given in the Annex.

3.2	Usage of narrow-band systems

The frequency bands 450.000 – 457.400 MHz and 458.400 – 460.000 MHz as well as 460.000 – 467.400 MHz and 468.400 – 470.000 MHz may be used in duplex or simplex operation.

In the case of duplex operation the base station shall transmit in the band 460 –  470 MHz and the mobile station shall transmit in the band 450 -  460 MHz.

In the case of simplex use within the preferential blocks, mobile station frequencies may be used on a preferential basis and base station transmitter frequencies may only be used in the band 460 – 470 MHz on a preferential basis, but the receiver frequencies of base stations in the  band 460 – 470 MHz cannot claim any protection.

Frequencies may be used under the conditions of a preferential frequency if the bandwidth of the emission is within the band limits of the preferential frequency blocks established in the Annex.

3.2.1 Preferential frequencies for narrowband systems (bandwidths < 25 kHz)

3.2.1.1 Distance between the base station and the border line equal to or less than 15 km.

Preferential frequencies may be used without co-ordination  if the field strength does not exceed a value of 34 dBμV/m/25 kHz at a height of 10 m above ground at a distance of 15 km inside the  affected country.

The propagation curves for analogous emissions with a time probability of 10% or with a time probability of 1%  for digital emissions shall be used.

3.2.1.2	Distance between the base station and the border line more than 15 km

Preferential frequencies may be used without co-ordination if the field strength does not exceed a value of 20 dBμV/m/25 kHz at a height of 10 m above ground at a distance of 50 km inside the affected country.

The propagation curves for analogous emissions with a time probability of 10% or with a time probability of 1%  for digital emissions shall be used.

3.2.2  Non-preferential frequencies for narrowband systems (bandwidths < 25 kHz)

Co-ordination requests on non-preferential frequencies may be limited by giving co-ordination status H to the co-ordination request with reference to the preferential right (4.2.3 of the Agreement (Berlin 2003)).

Non-preferential frequencies may be used without co-ordination with a neighbouring country if the field strength does not exceed a trigger value of 20 dBμV/m/25 kHz at a height of 10 m above ground at the border line.

The propagation curves for analogous emissions with a time probability of 10% or with a time probability of 1%  for digital emissions shall be used.

3.3	Frequencies for wideband systems (bandwidth = 1.25 MHz)

3.3.1	Distance between the base station and the border line equal to or less than 15 km

Frequencies may be used for wideband systems without coordination if the field strength does not exceed a value of 37 dBμV/m/1.25 MHz at a height of 10 m above ground at a distance of 15 km inside the affected country.

The propagation curves with a time probability of 1% shall be used.

3.3.2	Distance between the base station and the border line more than 15 km

Frequencies may be used for wideband systems without coordination if the field strength does not exceed a value of 20 dBμV/m/1.25 MHz at a height of 10 m above ground at a distance of 50 km inside the affected country.

The propagation curves with a time probability of 1% shall be used.

3.4	Shared frequencies

Shared frequencies may be used on the basis of bilateral agreements between affected countries or without co-ordination on a non-protected basis.

3.5	Protection for receivers

Protection for receivers on preferential frequencies can only be claimed under the following  conditions (see also Annex 1 of the “Agreement (Berlin 2003)”):

The reference transmitter with an effective radiated power of 16 dBW produces a field strength of max. 20 dBµV/m/25 kHz in a distance of 50 km (maximum cross-border range) from the border inside the other country.

The effective radiated power of the reference-transmitter has to be increased by the antenna gain of the receiver in the actual direction.

The propagation curves with a time probability of 10% are used.

3.6	Calculation method

The calculations of the interfering field strength are based on the “Agreement (Berlin 2003)” and shall be carried out with the official version of the HCM program.

4.	Administrative procedure

In derogation of  the “Agreement (Berlin 2003)”, the following special procedures are agreed:

Responses to notifications of preferential frequency assignments are not required.

The assignment of a preferential frequency shall be entered in the frequency register with co-ordination status P.

The assignment of frequencies to wideband systems fulfilling the conditions of paragraph 3.3.1 shall be entered in the frequency register with co-ordination status P and frequency category 7 (pending a future amendment of the “Agreement (Berlin 2003)” in this respect).

5.	Status of existing stations

This Agreement shall not apply to existing frequency utilisations agreed between administrations prior to this Agreement. Frequencies included in the frequency list that will be provided between administrations concerned for the frequency range 450.000 – 457.400 MHz and 458.400 – 460.000 MHz as well as 460.000 – 467.400 MHz and 468.400 – 470.000 MHz until end of February 2005, have to be protected until removal from service in accordance with their co-ordination status. Possible harmful interference caused by them shall be accepted.

Narrow-band stations, which have been included in the above mentioned frequency list, when new wideband systems with channel spacing of 1.25 MHz are introduced, shall be protected at their receiver antenna heights at a field strength level (Emax per 1.25 MHz) determined by the following formula:

Emax = 14 dBµV/m + 10 log(1250 (kHz) / channel spacing of narrow-band station (kHz))

6.	Status of existing Agreements

The Agreement “Vereinbarung über die Nutzung des Frequenzbereiches 450 – 470 MHz zwischen den Fernmeldeverwaltungen von Ungarn, der Tschechoslowakei und Österreich” (old agreement) entered into force on 1 January 1982 can be used by the administrations of Austria, the Czech Republic, the Slovak Republic and Hungary according to the following provisions until 31 December 2009:

- Frequency channels not completely falling into the own preferential frequency ranges of a country according to this new agreement and are in the frequency list mentioned under Article 5 of this agreement can be used in line with the old agreement.

- All other frequencies shall be used in accordance with this new agreement.

Concerning the frequency band 450.0 – 451.3 / 460.0 – 461.3 MHz  the “Agreement between the telecommunications administrations of Austria, Croatia, the Czech Republic, Hungary, the Slovak Republic and Slovenia concerning the allotment of preferential frequencies in the bands 410 – 420 / 420 – 430 MHz and  450.0 – 451.3 / 460.0 – 461.3 MHz (Vienna, 30 September 1994)” is replaced by this new agreement.

7.	Review

This Agreement can be revised  in light of administrative, regulatory or technical developments, especially in order to comply with relevant amendments of the “Agreement (Berlin 2003)”, at the proposal of any Signatory Administration with the agreement of all other Signatory Administrations.

In particular, once technical provisions of wideband systems with a channel spacing of 200 kHz have been clarified, this Agreement can be revised.

8.	Withdrawal

Any Administration may withdraw from this Agreement by the end of a calendar month by giving notice of its intention at least six months in advance. A declaration to that effect shall be addressed to all other Signatory administrations and to the managing administration of the “Agreement (Berlin 2003)“. Frequency assignments made within the framework of this Agreement prior to the date of entry into force of the withdrawal shall remain valid and be protected according to their status.

9.	Language of the Agreement

The original text of this Agreement exists in English and is retained at the managing administration of the “Agreement (Berlin 2003)“.

10.	Date of entry into force of the Agreement

This Agreement will enter into force at the date of its signature subject to confirmation by the Signatories. Such confirmation should be sent to the Austrian administration not later than 31 January 2005. The Austrian administration will inform the other Signatories accordingly. Confirmation letters will be annexed to this Agreement.

In the case that no confirmation is given by a certain administration, appropriate columns of the preferential frequency partitioning table which contain the relevant country name should be considered null and void.

After definitive entry into force of this Agreement, the Austrian administration will forward the original version of this Agreement to the managing administration of the Agreement (Berlin, 2003).

Done at Vienna, 3 December 2004

For the Austrian Administration

(Franz ZIEGELWANGER)

For the Czech Administration

(Jiri DUCHAC )

For the German Administration

(Thomas HEUTMANN )

For the Hungarian Administration

(Emilia PETRAS)

For the Italian Administration

(                                 )

For the Administration of Liechtenstein

(Kurt BÜHLER )

For the Slovak Administration

( Svetlana TOPOLSKA )

For the Slovenian Administration

(Iztok LAMPE)

For the Swiss Administration

( Konrad VONLANTHEN )